Article 1
Subject matter and scope

- This Regulation establishes requirements for the labelling of, and the provision of supplementary product information on, electric mains-operated commercial dishwashers of the following product categories:

- (a)	under-counter one-tank dishwashers;

- (b)	hood-type dishwashers.

This Regulation shall not apply to:

household dishwashers;

dishwashers designed exclusively for industrial purposes.

Article 2
Definitions

For the purposes of this Regulation, the following definitions shall apply:

- ‘commercial dishwasher’ means a machine which cleans, rinses, and optionally dries wash ware, such as dishware, glassware, cutlery, and other utensils connected to the preparation, cooking, arrangement or serving of food (including drinks), using chemical, mechanical, and thermal means; which is connected to electric mains and which is designed to be used principally for commercial (non-household, non-industrial) purposes as stated by the manufacturer in the Declaration of Conformity (DoC);

- ‘hood-type dishwasher’ means a manually loaded, programmable, hood-type, pass-through machine with typically one detergent-circulating zone and a fresh-water rinsing process;

- ‘household dishwasher’ means a machine which cleans and rinses tableware, and which is declared by the manufacturer in the Declaration of Conformity to comply with Directive 2014/35/EU of the European Parliament and of the Council or with Directive 2014/53/EU of the European Parliament and of the Council;

- ʻmains’ or ‘electric mains’ means the electricity supply from the grid of 230 (±10 %) volts of alternating current at 50 Hz;

- ‘point of sale’ means a location where commercial dishwashers are displayed or offered for sale, hire or hire-purchase;

- ‘under-counter one-tank dishwasher’ means a manually loaded, programmable, undercounter front loader with typically one detergent circulating zone and a fresh-water rinsing process.

For the purposes of the annexes, additional definitions are set out in Annex I.

Article 3
Obligations of suppliers

- Suppliers shall ensure that:

      - each commercial dishwasher is supplied with a printed label in the format as set out in Annex III;

the parameters of the product information sheet, as set out in Annex V, are entered into the product database;

if specifically requested by the dealer, the product information sheet shall be made available in printed form;

the content of the technical documentation, set out in Annex VI, is entered into the product database;

any visual advertisement for a specific model of commercial dishwasher contains the energy efficiency class and the range of energy efficiency classes available on the label in accordance with Annex VII and Annex VIII;

any technical promotional material concerning a specific model of commercial dishwasher, including on the Internet, which describes its specific technical parameters, includes the energy efficiency class of that model and the range of energy efficiency classes available on the label, in accordance with Annex VII;

an electronic label in the format and containing the information, as set out in Annex III, is made available to dealers for each commercial dishwasher model;

an electronic product information sheet, as set out in Annex V, is made available to dealers for each commercial dishwasher model.

The energy efficiency class is defined in Annex II and shall be calculated in accordance with Annex IV.

Article 4
Obligations of dealers

Dealers shall ensure that:

  - each commercial dishwasher, at the point of sale, including at trade fairs, bears the label provided by suppliers in accordance with point 1(a) of Article 3, with the label being displayed for built-in commercial dishwashers in such a way as to be clearly visible, and for all other commercial dishwashers in such a way as to be clearly visible on the outside of the front or top of the commercial dishwasher;

in the event of distance selling, the label and product information sheet are provided in accordance with Annexes VII and VIII;

any visual advertisement for a specific model of commercial dishwasher contains the energy efficiency class of that model and the range of energy efficiency classes available on the label, in accordance with Annex VII;

any technical promotional material concerning a specific model of commercial dishwasher, including on the Internet, which describes its specific technical parameters, includes the energy efficiency class of that model and the range of energy efficiency classes available on the label, in accordance with Annex VII.

Article 5
Obligations of internet hosting platforms

Where a hosting service provider referred to in Article 14 of Directive 2000/31/EC allows the direct selling of commercial dishwashers through its Internet website, the service provider shall enable the showing of the electronic label and electronic product information sheet provided by the dealer on the display mechanism in accordance with the provisions of Annex VIII and shall inform the dealer of the obligation to display them.

Article 6
Measurement methods

Information to be provided pursuant to Articles 3 and 4 shall be obtained by reliable, accurate and reproducible measurement and calculation methods, which take into account the recognised state-of-the-art measurement and calculation methods set out in Annex IV.

Article 7
Verification procedure for market surveillance purposes

Member States shall apply the verification procedure laid down in Annex IX to this Regulation when performing the market surveillance checks referred to in paragraph 3 of Article 8 of Regulation (EU) 2017/1369.

Article 8
Review

The Commission shall review this Regulation in the light of technological progress and present the results of this review including, if appropriate, a draft revision proposal, to the Consultation Forum no later than [OP – please insert the date - six years after day of entry into force of this Regulation].

The review shall in particular assess the following:

  - the improvement potential with regard to the energy consumption, functional and environmental performance of commercial dishwashers;

  - the extension of the scope of this Regulation to include product categories that are currently excluded according to Article 1(2)(a);

  - the revised version of EN IEC 63136 and any resulting implications for the functional requirements of commercial dishwashers;

  - an assessment of the evolution of consumer behaviour and of the penetration rate of commercial dishwashers in EU Member States;

the effectiveness of existing measures in achieving changes in end-user behaviour in purchasing more energy and resource efficient appliances and using more energy and resource efficient programmes;

the appropriateness of setting additional resource efficiency requirements for products in accordance with the objectives of the circular economy, including whether more spare parts should be included.

Article 9
Entry into force and application

This Regulation shall enter into force on the twentieth day following that of its publication in the Official Journal of the European Union.

It shall apply from XX.XX.XXXX [18 months after adoption of this Regulation].

This Regulation shall be binding in its entirety and directly applicable in all Member States.